# Data Collection w/ Macchina P1

# Guide

# TABLE OF CONTENTS

1.	Setup	3

2.	Installation	3

3.	Access	4

4.	Internet	4

5.	Updates & Pins	6

6.	Data Capture	7

7.  Bitrate	8

8. Python 	8

9. Data Viewing	10

Step One:  Setup

- The macchina p1 consists of two main parts: Pocket Beagle and the Macchina pictured here

  - The two halves however come put together already saving some initial setup time

  - The Macchina p1 comes with a Wifi dongle and a micro usb along with a sd card

    - The macchina is usb powered

- Proceed by plugging the micro usb into the top half of the Macchina P1 and the wifi dongle into the lower half

Step Two:  Installs

- After getting everything plugged in a bootable SD card can be created which would then be used by the macchina however macchina comes with a preinstalled image which does suffice for the most part

- However if it is decided to create a new bootable sd card the link below will allow for the latest images to be downloaded

  - https://beagleboard.org/latest-images

- A tutorial of how to create the bootable sd drive can be found here

Step Three:  Access

- At this point one should connect to the device for the first time.

  - This can be done either via a preferred terminal or via the web browser

    - There are two options for this 192.168.7.2 & 192.168.6.2

    - 192.168.6.2  should only be used if .7.2 does not allow for connection

Password

- At this point it's possible to change your password

- In order to do this the following commands make this possible

  - sudo passwd root

    - *Note the red is where the user denotes where the user adds their own input

Step Four: Internet

- At this phase, it is necessary to begin connecting to the internet and where issues may begin to arise

- The commands used to connect to begin searching for wifi access points should go as follows:

  - sudo connmanctl

  - connmanctl> disable wifi

  - Disabled wifi

  - connmanctl> enable wifi

  - Enabled wifi

  - connmanctl> scan wifi

  - Scan completed for wifi

  - connmanctl> services

- Upon running these commands a list of available wifi options should be displayed

- Avoid using local or guest wifi options that require an authorization page!

  - As the Macchina will not be able to bypass the page and simply be stuck in a loop

    - If your macchina does find itself in this loop, using a personal hotspot like from a phone will allow it to connect to the internet

  - After deciding on a Wifi to connect to the following commands should be ran

    - connmanctl> agent on

    - connmanctl> connect wifi_xxxxxx_xxxxxx_managed_psk

    - Passphrase? xxxxxxxxxxx

    - connected wifi_xxxxxx_xxxxxx_managed_psk

    - connmanctl> quit

  - Running the command ifconfig will confirm wifi connectivity

  - If wifi connectivity has failed ssh over to the beagle bone via previously mentioned  putty and run the following

    - sudo connmanctl disable wifi

    - sudo connmanctl enable wifi

    - iwconfig wlan0

    - ifconfig wlan0

Step Five: Updates

- It is crucial that step four has been completed otherwise updates will fail

- The commands for completing the update go as follows

  - sudo apt update

  - sudo apt upgrade

- In the case of the previously mentioned loop with the Wifi the macchina appears to be updating but several will just fail for no apparent reason

- If this is the case then connecting to a different wifi source and repeating step 4 may be necessary

- The next steps of Step Five  will only ever be performed once upon completion

- Verify the following files are available

  - ls -lt /lib/firmware/PB-CAN0-00A0.dtbo

  - ls -la /lib/firmware/PB-CAN1-00A0.dtbo

    - If one or both of these files don’t exit downloading a new version of the SD drive is necessary

- Next edit the files via nano

  - sudo nano /boot/uEnv.txt

- Replace the lines in white with those in red in

  - #uboot_overlay_addr4=/lib/firmware/<file4>.dtbo → uboot_overlay_addr4=/lib/firmware/PB-CAN0-00A0.dtbo

  - #uboot_overlay_addr5=/lib/firmware/<file5>.dtbo → uboot_overlay_addr5=/lib/firmware/PB-CAN1-00A0.dtbo

- Finally reboot the system

  - sudo reboot

Step Six: Data Capture

- At this point it becomes time to get the first can data samples this can be done by the following commands

  - sudo ip link set can0 type can bitrate 250000 listen-only on

    - Note bitrate will vary vehicle to vehicle

      - The generic bitrate did not work for the vehicle simulator

      - Other suggested bitrates: 500,000 | 125,000 | 33,333

        - If any of the suggested bitrates work continue through step 6 if not skip ahead to step 7

  - sudo ifconfig can0 up

  - candump -cae can0,0:0,#FFFFFFFF

    - An output of the data similar to the picture to the right a constant stream of output

    - If no output is given the image will look similar to the image below consisting of a continuous loading

    - If you find yourself with a continuous loading loop proceed to step 7 if it was successful however please move on to step 8

Step Seven: Bitrate

- This phase involves utilizing Vehicle Spy the software itself has a free trial version that will suffice as only needs to be used once

- Simply downloading Vehicle Spy trial and loading the software will produce various forms of data automatically as its not bit rate nor can specific in the data it captures

- After a few minutes of capturing outputting the .csv fuzzer file will give a list of varying bitrates an example of the file output is included to the left

  - OBD_Fuzzer_V2A032_12-01-2021_7-37-22_pm.csv

- Other options to discover the baud rate of your personal vehicle include actually calling your car manufacture about the baud rate/bitrate of your vehicle make and model

Step Eight: Python

- To begin download the following file via the command below into putty or the terminal

  - wget -P ~/scripts https://gist.githubusercontent.com/kenny-macchina/cf47daf663f9091460fc3454f5e38de9/raw/676f2a36f300fa8e5c9c43c05a7bc18af2479b5c/p1_data.py

- After grabbing the package installing the package is the next step however without updating python the install will fail if python is not up to version 3

  - To check version of python

    - python –version

  - To update python run the following

    - sudo apt update

    - sudo apt install python3.9

      - *Replace 3.9 with whatever version python is currently on

- After performing the update you can now begin the install of the previously grabbed script

  - curl https://bootstrap.pypa.io/get-pip.py -o get-pip.py

  - python get-pip.py

  - sudo reboot now

- Upon successful completion being installing the pip package with the following commands

  - curl https://bootstrap.pypa.io/get-pip.py -o get-pip.py

  - python get-pip.py

  - sudo reboot now

- Finally install the CAN library which will allow for the data to be analyzed and converted into a csv file

  - pip install python-can

  - sudo reboot now

Step Nine: Viewing the Data

- With python set up the user can once again begin collecting data this time utilizing the python script in the home directory run the following:

  - python scripts/p1_data.py

    - The image left shows a successful output

  - python3 -m http.server 1337

    - The second command outputs the csv file and its data in a server that can then be more easily viewed an example of this is shown below